EROSION and SEDIMENTATION CONTROL PLAN PRELIMINARY REVIEW CHECKLIST

The following items shall be incorporated with respect to specific site conditions, in an erosion & sedimentation control plan:

NPDES Construction Stormwater General Permit NCG010000

______   Designation on the plans where the 7 or 14 day ground stabilization requirements apply per Part II.E.1 of the permit.

______   Design of basins with one acre or more of drainage area for surface withdrawal as per Part II.B.8 of the permit.

## LOCATION INFORMATION

______	Project location & labeled vicinity map (roads, streets, landmarks)

______	North arrow and scale

______	Identify River Basin.

______   Provide a copy of site located on applicable USGS quadrangle and NRCS Soils maps if it is in a River Basin with Riparian Buffer requirements.

GENERAL SITE FEATURES (Plan elements)

_____	Property lines & ownership ID for adjoining properties

_____	Existing contours (topographic lines)

_____	Proposed contours

_____	Limits of disturbed area (provide acreage total, delineate limits, and label). Be sure to include all access to measures, lots that will be disturbed, and utilities that may extend offsite.

_____	Planned and existing building locations and elevations

_____	Planned & existing road locations & elevations, including temporary access roads

_____	Lot and/or building numbers

_____	Hydrogeologic features:  rock outcrops, seeps, springs, wetland and their limits, streams, lakes, ponds, dams, etc. (include all required local or state buffer zones and any DWQ Riparian Buffer determinations)

_____	Easements and drainageways, particularly required for offsite affected areas. Include copies of any recorded easements and/or agreements with adjoining property owners.

_____	Profiles of streets, utilities, ditch lines, etc.

_____	Stockpiled topsoil or subsoil locations

_____	If the same person conducts the land-disturbing activity & any related borrow or waste activity, the related borrow or waste activity shall constitute part of the land-disturbing activity unless the borrow or waste activity is regulated under the Mining Act of 1971, or is a landfill regulated by the Division of Waste Management.  If the land-disturbing activity and any related borrow or waste activity are not conducted by the same person, they shall be considered separate land-disturbing activities and must be permitted either through the Sedimentation Pollution Control Act as a one-use borrow site or through the Mining Act.

______	Location and details associated with any onsite stone crushing or other processing of material excavated.  If the affected area associated with excavation, processing, stockpiles and transport of such materials will comprise 1 or more acres, and materials will be leaving the development tract, a mining permit will be required.

______	Required Army Corps 404 permit and Water Quality 401 certification (e.g. stream disturbances over 150 linear feet)

#### EROSION & SEDIMENT CONTROL MEASURES (on plan)

______	Legend (provide appropriate symbols for all measures and reference them to the construction details)

______	Location of temporary measures

______	Location of permanent measures

______	Construction drawings and details for

temporary and permanent measures. Show measures to scale on plan and include proposed contours where necessary. Ensure design storage requirements are maintained through all phases of construction.

_____	Maintenance requirements for measures

_____	Contact person responsible for maintenance

### SITE DRAINAGE FEATURES

_____	Existing and planned drainage patterns (include off-site areas that drain through project and address temporary and permanent conveyance of stormwater over graded slopes)

_____	Method used to determine acreage of land being disturbed and drainage areas to all proposed measures (e.g. delineation map)

_____	Size, pipe material and location of culverts and sewers

_____	Soil information:  type, special characteristics

_____	Soil information below culvert storm outlets

_____	Name and classification of receiving water course or name of municipal operator (only where stormwater discharges are to occur)

### STORMWATER CALCULATIONS

_____	Pre-construction runoff calculations for each outlet from the site (at peak discharge points). Be sure to provide all supporting data for the computation methods used (rainfall data for required storm events, time of concentration/storm duration, and runoff coefficients).

_____	Design calculations for peak discharges of runoff (including the construction phase & the final runoff coefficients for the site)

_____	Design calcs for culverts and storm sewers (include HW, TW and outlet velocities)

_____	Discharge and velocity calculations for open channel and ditch flows (easement & rights-of-way)

_____	Design calcs for cross sections and method of stabilization for existing and planned channels (include temporary linings). Include appropriate permissible velocity and/or shear stress data.

_____	Design calcs and construction details for energy dissipaters below culvert and storm sewer outlets (include stone/material specs & apron dimensions). Avoid discharges on fill slopes.

_____	Design calcs and dimension of sediment basins (note current surface area and dewatering standards as well as diversion of runoff to the basins). Be sure that all surface drains, including ditches and berms, will have positive drainage to the basins.

VEGETATIVE STABILIZATION

_____	Area & acreage to be stabilized with vegetation

_____	Method of soil preparation

_____	Seed type & rates (temporary & permanent)

_____	Fertilizer type and rates

_____	Mulch type and rates (include mulch anchoring methods)

NOTE:	Plan should include provisions for groundcover in accordance with NPDES Construction Stormwater General Permit NCG010000.

FINANCIAL RESPONSIBILITY/OWNERSHIP FORM

_____	Completed, signed & notarized FR/O Form

_____	Accurate application fee payable to NCDEQ ($65.00 per acre rounded up the next acre with no ceiling amount)

_____	Certificate of assumed name, if the owner is a partnership

_____	Name of Registered Agent (if applicable)

_____	Copy of the most current Deed for the site. Please make sure the deed(s) and ownership information are consistent between the plan sheets, local records and this form.

_____	Provide latitude & longitude (in decimal degrees) at the project entrance.

_____	Two hard-copies of the plans (some regional offices require additional plans or multiple sizes; please contact the regional coordinator prior to such submittal.)

NOTE:	For the Express Permitting Option, inquire at the local Regional Office for availability. Express Reviews are performed by appointment only.

NARRATIVE AND CONSTRUCTION SEQUENCE

_____	Narrative describing the nature & purpose of the construction activity.

_____	Pre-construction conference, if requested.

_____	Construction sequence related to erosion and sediment control (including installation of critical measures prior to the initiation of the land-disturbing activity & removal of measures after areas they serve are permanently stabilized). Address all phases of construction and necessary practices associated with temporary stream bypasses and/or crossings.

_____	Bid specifications related only to erosion control
                                                                               rev. 10122020